Fully Automatic Glue-Free Coreless Rewinder ZRF-D600

Plan sketch：

Basic Use：

It is mainly used for glue-free rewinding of materials such as kraft paper, wallpaper and automotive films, etc.

Product characteristics：

- Hokawa motion control system, servo control for winding, servo control for active traction.

- The automatic cylinder feeding system is adopted, which is convenient, fast and labor-saving.

- Automatic tension winding and unwinding, with stable tension and smooth start/stop; servo motor torque winding, with stable and precise tension, exquisite winding quality, and flat end surfaces.

- Without core insertion, the end is automatically sealed with adhesive tape, and the control system is from Hokkaido. Thus, the automatic and seamless re-rolling function without adhesive and without core is achieved.

- Action characteristics: Cut after clamping, attach tail label, retract and contract to unload, complete one action and then deliver the material for the next feeding, then the roll extends to clamp the material, the roll is a clamping type roll, achieving continuous and effective operation.

Main technical parameters：

| Model | SWF-D600 |
| --- | --- |
| paper core | Collecting the answers |
| Unwinding master roll | 500mm Max. Unwind Roll Diameter 500mm |
| Rewinding diameter | 20-150mm |
| Rewind width | W（0-600）mm |
| Production speed | Mechanical Line Speed 120m/min |
| power supply | 220V 50/60HZ4KW |
| air pressure | 5KG |
| Machine size | 2100*1500*1400mm |
| Net Weight | 800KG |

Configuration parameters：

| - Model | SWF-D600 |
| --- | --- |
| - Touch screen | 7-inch MCGS HMI |
| - Rack wall panel | 36mm |
| - Movement controller | HCFA |
| - Active traction servo motor | HCFA |
| - Active traction servo driver | HCFA |
| - Receiving roll servo motor | HCFA |
| - Rolling shutter servo driver | HCFA |
| - Solenoid valve | ASICKO |
| - Cylinder | AIRTAC |
| - Linear slide | HIWIN |
| - Magnetic switch | AIRTAC |
| - Unroll 3-inch air expansion shaft | Taiwan,China |
| - bearing | NSK |
| - Traction shaft | Precision grinding roller |
| - Compression roller | Silicone package |
| - Transitional roller | European standard aluminum guide wheels |