Fully Automatic Glue-Free Rewinding Machine ZRF-1300

Equipment layout：

Basic Use：

Mainly used for glue-free rewinding of materials such as aluminum foil tape, copper foil tape, automotive paint protection film, advertising paper, color-changing film and kraft paper.

Product characteristics：

- Adopts PC-based motion control system; the whole machine is driven by 11 servo motors, eliminating the instability of pneumatic control and improving equipment stability and efficiency. (Servo-controlled winding, servo-controlled active traction, servo-controlled label pulling, servo-controlled adhesive application, servo-controlled reversing & shaft switching, servo-controlled discharging, servo-controlled winding pressure roller, servo-controlled cutting knife, servo-controlled finishing, servo-controlled finishing labeling, servo-controlled paper core feeding).

- Adopts a high-precision indexer drive positioning structure with 4-axis automatic switching, enabling non-stop production and improving production efficiency.

- Equipped with automatic labeling at winding and finishing sections, ensuring flat joints for easy subsequent processing.

- Automatic smoothing rollers installed at winding and finishing sections ensure fast sealing without material wrinkles.

- Adopts our self-developed zero-friction zero-contact rewinding structure, replacing traditional shaft-locking mechanism; the machine runs silently without material damage.

- Servo-driven automatic shifting pressure roller automatically follows material diameter changes, maintaining a consistent gap between pressure roller and winding shaft for flatter rewinding.

- Adopts hydraulic brake automatic loading to prevent master roll paper core deformation and shaft threading difficulty, ensuring convenient, fast and labor-saving operation.

- Equipped with automatic alarm and shutdown for material absence to reduce material waste.

- Built-in automatic deviation correction to avoid defective products caused by uneven master roll materials.

- Fitted with a material receiving platform for easy material threading, improved efficiency and reduced waste.

- Servo modules drive pulling/releasing of sealing labels; length and position are freely adjustable for convenient setup and high positioning accuracy.

- Mobile discharge table stores 20–40 finished rolls and can be moved freely for easy subsequent processing.

- Automatic paper core loading, servo-driven core threading, automatic adhesive application, automatic finishing & sealing labeling, automatic cutting and automatic discharging.

- Automatic tension control for winding/unwinding ensures stable tension, flat lamination and smooth start/stop. Edge correction guarantees flat rewinding.

- Servo-driven winding with digital linear tension increase (tension calculated per revolution) enables smooth transition of winding/unwinding force.

- Servo motor-driven feeding ensures accurate length calculation. Servo torque-controlled winding delivers stable, precise tension for high-quality rolls with flat end faces.

- Unwinding uses shaftless hydraulic brake for easy/fast loading/unloading; equipped with hydraulic cylinder lifting, automatic deviation correction and magnetic powder brake roll-diameter-based automatic tension control for precise torque regulation.

- Rotary cutting with tungsten steel circular knife + automatic knife cleaning system; durable blade with neat cuts.

- Alarms and shutdowns for material absence, material breakage and paper core shortage.

- Equipped with HCFA motion control system, supporting one-click start and easy specification changeover.

- Enables unmanned production; one operator can manage multiple machines, significantly reducing labor costs and improving efficiency.

Main Technical Parameters：

| Model | SWF-1300 |
| --- | --- |
| paper core | (Customizable & freely replaceable components） 76 |
| Unwinding master roll | 800mm(MAX) |
| Rewinding diameter | 100-160mm |
| Rewind width | W（800-1300）mm |
| Production speed | 200m/min 4-5pcs/min（15m） subject to rewinding length |
| power supply | 380V 50/60HZ/15KW |
| air pressure | 5KG |
| Machine size | 5239*5222*2082mm |
| Net Weight | 3500KG |

Configuration parameters：

| - model | SWF-1300 |
| --- | --- |
| - touch screen | 14-inch MCGS HMI |
| - Rack wall board | 36mm |
| - Motion controller | HCFA |
| - Active traction servo motor | HCFA |
| - Active traction servo driver | HCFA |
| - Reversing Servo Motor | HCFA |
| - Reversing Driver | HCFA |
| - Adhesive Application Servo Motor | HCFA |
| - Adhesive Application Driver | HCFA |
| - Label Pulling Servo Motor | HCFA |
| - Label Pulling Driver | HCFA |
| - Labeling Servo Motor | HCFA |
| - Labeling Driver | HCFA |
| - Discharge servo motor | HCFA |
| - Discharge driver | HCFA |
| - Coiling press roller servo motor | HCFA |
| - Winding press roll driver | HCFA |
| - Traction Servo Motor | HCFA |
| - Traction Driver | HCFA |
| - Cutter servo motor | HCFA |
| - Cutter driver | HCFA |
| - Winding Servo Motor | HCFA |
| - Winding Servo Drive | HCFA |
| - Pipe threading servo motor | HCFA |
| - Pipe threading servo driver | HCFA |
| - Finishing Servo Motor | HCFA |
| - Finishing Servo Driver | HCFA |
| - Four-axis flip divider | JUJIU |
| - Motor | MaiLi |
| - Cutter motor | MaiLi |
| - Reducer | SMETRE |
| - Magnetic powder brake | WREZUN |
| - Air switch | CHINT |
| - Switching power supply | Mingwei China |
| - Linear guide | KENIC |
| - Deviation Correction System | Youdao |
| - Photoelectric sensor | BOJKE |
| - Material detection sensor | BOJKE |
| - Proximity Switch | Yangming |
| - Contactor | Schneider |
| - Relay | IDEA |
| - Button switch | Schneider |
| - Tri-color warning light | TAYB |
| - Solenoid valve | ASICKO |
| - Cylinder | AIRTAC |
| - Linear slide | HIWIN |
| - Magnetic switch | AIRTAC |
| - Unwinding, waste collection 3 inch gas expansion shaft | TAIWAN CHINA |
| - bearing | NSK |
| - Traction shaft | Nano coating for anti-sticking |
| - Compression roll | Package of silica gel |
| - transition roller | Hard Anodizing |
| - scalpel | 110MM tungsten steel round knife |