Fully Automatic Tape Rewinder ModelZRF-600

Equipment Layout：

Basic Use：

It is mainly used for laminating and rewinding adhesive tape.

Product characteristics：

- Matsushita PLC is adopted, with servo motor for material feeding and automatic shaft replacement.

- The winding is smooth, the start and stop are stable, and the edge correction is controlled.

- Paper drop, automatic threading, automatic cutting, automatic finishing, automatic discharging,

- Automatic tension unwinding, stable tension and smooth fit. Start and stop the volume smoothly.

- The servo motor pulls the material, and the length is calculated accurately.

- The unwinding adopts manual expansion shaft, which is convenient and fast for loading and unloading.

- Rewind with adhesive surface or peel off the base paper

- Stop when there is no material alarm, material break alarm, no paper tube alarm and bottom paper break alarm.

- One key start, easy operation and unmanned whole process.

- Servo motor torque winding, stable and precise winding tension, exquisite winding, flat end surface.

Main technical parameters：

| Model | SHF-600 |
| --- | --- |
| Paper Core | Customized according to customer requirements |
| Unwinding Master Roll | Max. Roll Diameter: 550mm |
| Rewinding Diameter | within 60mm |
| Rewind Width | Φ（50-600）mm |
| Production Speed | 6-10pcs/MIN Subject to rewinding length |
| Power Supply | 1P220V 50/60HZ/4KW |
| Air Pressure | 5KG/CM2 |
| Machine Size | 1800*2150*1250mm |
| Net Weight | 550KG |

Configuration parameters：

| - Model | SHF-600 |
| --- | --- |
| - Touch Screen | MCGSTPC |
| - PLC | - MATSUSHITA |
| - Expansion Module | - MATSUSHITA |
| - Servo Motor | - JINTIAN |
| - Axis Changer Servo Drive | - JINTIAN |
| - Servo Motor | - SHILIN |
| - Pulling Servo Drive | - SHILIN |
| - Winding Servo Motor | - SHILIN |
| - Winding Servo Drive | - SHILIN |
| - Core Loading Speed Regulating Motor | - SANLY |
| - Tail Winding Constant Speed Motor | - SANLY |
| - Tension Controller | - IMPQCI |
| - Reducer | - HASI Mai |
| - Turbine Reducer | - WANYU |
| - Edge Corrector | - ARISE |
| - Magnetic Powder Clutch | - YOSEEN |
| - Magnetic Powder Brake | - YOSEEN |
| - Air Switch | - CHINT |
| - Switching Power Supply | - MINGWEI |
| - Edge Correction Power Supply | - MINGWEI |
| - Photoelectric Sensor | - DCF |
| - Material Detection Sensor | - DCF |
| - Incoming Proximity Switch | - CNTD |
| - Discharge Proximity Switch | - CNTD |
| - Contactor | - SCHNEIDER |
| - Relay | - SCHNEIDER |
| - Button Switch | - KNDELEK |
| - Tri-color warning light | - TAYB |
| - Solenoid Valve | - China STNC |
| - Cylinder | - STAR |
| - Rodless Cylinder | - JIASO |
| - Linear Slide | - HIWIN |
| - Magnetic Switch | - AIRTAC |
| - Bearing | - NSK，HRB |
| - Drive Spindle | - Embossed Teflon surface anti-stick treatment |
| - Nip Roll | - Silicone lagging (non-adhesive) |
| - Unwind Expanding Shaft | - Lug Type Manual Expanding Shaft |
| - Waste Rewind Expanding Shaft | - Lug Type Manual Expanding Shaft |
| - Transition Roller | - Teflon Coated Knurled Surface Treatment (to prevent material slipping and adhesive sticking) |