Fully Automatic Rewinding Machine with Core ZRF-600DY

- Equipment Layout：

- The specific parameters are as follows：

| SWF-600DY | Fully automatic glueless rewinding machine with Core - suitable for various glueless materials such as: advertising paper, kraft paper, wallpaper, etc. |  |  |
| --- | --- | --- | --- |
| Abstract | Specification details | Quantity | Brand |
| The applicable range of this machine |  |  |  |
| Effective discharge width | 300-600mm |  |  |
| Maximum discharge diameter | 800mm |  |  |
| Unroll the paper core | 3inch |  |  |
| The inner diameter of the reel | 26mm（can change） |  |  |
| Maximum diameter of the roll | 80mm |  |  |
| Mechanical speed | 0-150m/min |  |  |
| Power supply | 380V 50HZ |  |  |
| Air source | 5KG(customer self-provided) |  |  |
| Discharge configuration |  |  |  |
| Discharge seat | Extended spooling of 3 inches | 1 set |  |
| Brake control system | Automatic control of cone taper for unwinding tension | 1 set |  |
| Correction system | CPC automatic deviation correction | 1 set | EURDOW |
| Examination paper configuration |  |  |  |
| Collect the scroll | 26mm air expansion shaft (replaceable) | 1 set |  |
| Reel motor | 1KW servo motor | 1 set | HCFA |
| Traction motor | 1.8KW servo motor | 1 set | HCFA |
| Traction reducer | Straight gear planetary reducer | 1 set | SMITECH |
| Rolling tension system | Motion controller linear control | 1 set | HCEA |
| Rolling shutter mechanism | 1KW servo winding machine | 1 set |  |
| Final labeling configuration |  |  |  |
| Sealing label | Servo control labeling length (adhesive tape) | 2 sets |  |
| Labeling motor | 750W servo motor | 1 set | HCFA |
| Labeling reducer | Straight gear planetary reducer | 1 set |  |
| Cutting knife mechanism | Tungsten steel round knife rotary cutting (with blade washing function) | 1 set |  |
| Cutting blade motor | 30W DC brushless motor | 1 set | MAIOLI |
| Electrical Control Department Configuration |  |  |  |
| Control system | Motion control system (One-click start, easy to change specifications) | 1 set | HCFA |
| Operation panel | Set production parameters and tension control parameters | 1 set | 10-inch MCGS HMI |
| Control box | Embedded control box with push-button panel | 1 set |  |
| Other configurations |  |  |  |
| Guide roller | Surface hard coating |  |  |
| Random data |  |  |  |
| Chinese and English instructions | Two copies each in electronic format and paper format | 2 sets |  |
| Operation Video |  | 1 set |  |
| Electrical wiring diagram | Two copies each in electronic format and paper format | 2 sets |  |
| Gas and Liquid Principle Diagram | Two copies each in electronic format and paper format | 2 sets |  |
| List of vulnerable parts and spare parts | Two copies each in electronic format and paper format | 2 sets |  |
| Common tools |  | 1 set |  |