Dual-Shaft Horizontal Cutting Machine ZRS-D1300

- Equipment Layout：

- Basic Use：

- Mainly used for slitting materials such as dust-proof adhesive rolls, dust-removal films, tear-free dust removal paper rolls, PCB dust removal paper, lint removal paper rolls, PP dust removal rolls and PE dust removal rolls.

- Product Characteristics:

- Equipped with HCFA motion control system, featuring accurate parameters and high speed.

- Main spindle driven by 1500W three-phase asynchronous motor; circular blade driven by 750W servo motor with planetary reducer.

- Cross travel adopts HIWIN 2020 ball screw + 25 linear guide, driven by 750W servo motor; positioning tolerance within 1000mm: ±0.02mm.

- Blade feed adopts HIWIN 2005 ball screw + 25 linear guide, driven by 750W servo motor, ensuring feeding strength and accuracy.

- Waste pusher adopts HIWIN 1610 ball screw, driven by 400W servo motor, ensuring accurate feeding position.

- Material ejector adopts HIWIN 2005 ball screw + 25 linear guide, driven by 750W servo motor, preventing material damage.

- Blade holder and cross travel components undergo QBQ surface treatment, enhancing hardness, wear resistance, corrosion resistance and reducing deformation.

- 10-inch main touch screen for simple and intuitive operation.

- Integrated lighting: light for electrical cabinet door open, light for machine cabinet door open, operation lighting.

- Max. diameter of circular blade: 160mm, with stainless steel protective blade housing.

- Manual height adjustment at feeding end to adapt to different finished product diameters; cylinder-driven feeding.

- Automatic front/rear waste discharge; waste conveyed via inclined line for packaging.

- Finished products discharged automatically and orderly, directly connectable to packaging equipment.

- High-speed rotating expanding chuck with stable clamping, no damage to paper core; no slipping during slitting to prevent blade chipping.

- Designed for inline connection behind automatic rewinding machines, enabling automatic loading, slitting, waste discharge and orderly finished product discharge, reducing labor costs and adapting to automated production lines.

- Main Technical Parameters：

| Model | ZRS-D1300 |
| --- | --- |
| paper core | Customizable (interchangeable) 38/50/76 mm |
| Min. Slitting Width | 0.5-1300mm |
| Max. Slitting Diameter | 80MM |
| Max. Diameter of Slitting Blade | 160mm |
| HMI Operation | 10寸/inch |
| power supply | 380V(220V customizable), 50/60HZ/5KW |
| air pressure | 5KG |
| Machine size | 4000*1550*1600 |
| Net Weight | 1.2T |

- Configuration Parameters:

| Model | SWS-D1300 |
| --- | --- |
| Main Touch Screen | 10寸/inch HCFA |
| Discharge Conveyo | 200w |
| Rack Wall Plate | 80×80 square tube + 20mm steel plate |
| Motion Controller | HCFA |
| Main Spindle Three-Phase Asynchronous Motor | 1500W HCFA |
| Main Spindle Inverter | 5000W HCFA |
| Circular Blade Servo Motor | 750W HCFA |
| Circular Blade Driver | 750W HCFA |
| Cross Travel Servo Motor | 750W HCFA |
| Cross Travel Driver | 750W HCFA |
| Blade Feed Servo Motor | 750W HCFA |
| Blade Feed Driver | 750W HCFA |
| Pusher Servo Motor | 400W HCFA |
| Pusher Driver | 400W HCFA |
| Ejector Servo Motor | 750W HCFA |
| Ejector Driver | 750W HCFA |
| Waste Inclined Conveyor | 120W |
| Planetary Reducer | Smetre |
| Air Switch | chint |
| Switching Power Supply | Mingwei |
| Photoelectric Sensor | BOJKE |
| Proximity Switch | Yangming |
| AC Contactor | Schneider |
| Relay | IDEA |
| Push Button Switch | Schneider |
| 三色报警灯 Three-Color Alarm Lamp | 台邦TʌYB |
| Lighting | Waterproof LED |
| Blade Feed Linear Guide | HIWIN 25 |
| Blade Feed Ball Screw | HIWIN 2005 |
| Cross Travel Linear Guide | HIWIN 25 |
| Cross Travel Ball Screw | HIWIN 2020 |
| Pusher Ball Screw | HIWIN 1610 |
| Ejector Linear Guide | HIWIN 25 |
| Ejector Ball Screw | HIWIN 2010 |
| Solenoid Valve | asicko |
| Cylinder | AIRTAC |
| Linear Guide Rail | HIWIN |
| Magnetic Switch | AIRTAC |
| Main Spindle Chuck3-inch | DC53 |
| Bearing | NSK |
| Machine Body | Imported baking paint |
| Circular Blade | Customizable per material, max. 160mm |
| Toolbox | 1set |