Automatic 4-shaft cutting machine ZRT-1300-G4

Basic Use：

This machine is designed for slitting narrow-width adhesive tapes, including electrical tape, washi tape, masking tape, double-sided adhesive tape and other adhesive materials. Features: Four spindles perform slitting simultaneously to greatly boost production efficiency; each circular cutter can be adjusted independently.

Product characteristics：

- Equipped with VECON PLC control system, supporting 12 working modes for cutting products of different widths. Users can freely select the starting and cutting end positions, with three adjustable cutter feeding speeds for slitting.

- 2.Main spindle driven by a 5500W Shihlin frequency converter; circular cutter driven by a 3700W Shihlin frequency converter. The system will immediately halt upon alarm to prevent cutter breakage.

- 3，Cross travel mechanism adopts HIWIN ball screw and linear guide, controlled by a 1000W Shihlin servo motor. Positioning tolerance within 1000mm travel: ±0.02mm.

- 4.Cutter feeding unit fitted with HIWIN ball screw and linear guide, driven by a 1000W servo motor to guarantee stable feeding force and high cutting precision.

- 5.Swing angle unit uses HIWIN ball screw with stepper motor control. Adjustable swing angle range: -9.5° ~ +9.5°.

- 6.QBQ surface treatment applied to cutter base and cross travel components to improve hardness, wear resistance, corrosion resistance and anti-deformation performance.

- 7.10-inch main touch screen for simple and intuitive operation.

- 8.Stainless steel diversion pipeline drainage system avoids silicone oil splashing during operation.

Main technical parameters：

| Model | ZRT-1300-G4 |
| --- | --- |
| paper core | 25/38/50/76=4strips Customizable per customer requirements（interchangeable）available sizes:25/38/50/76=4 sets |
| Cutting Width（Min.）（Max.） | 0.5-1300mm |
| Max. Cutting Material Diameter | 150mm（Max material thickness 45mm; maximum spindle diameter varies by spindle model） |
| Max. Circular Cutter Diameter | 180mm |
| HMI Touch Screen | 10=inch |
| power supply | 380V（customizable 220V） 50/60HZ/8.8KW |
| air pressure | 5KG |
| Machine size | L2700 × W1100 × H1500 mm |
| Net Weight | 2T |

- Configuration parameters：

| Item | Specification Configuration |
| --- | --- |
| Model | ZRT-1300-G4 |
| Main Touch Screen | 10-inch |
| Frame Wall Plate | thickness 28 mm |
| PLC Controller | VECON |
| Main Spindle Motor | Three-phase asynchronous motor/ KETEMAI brand |
| Main Spindle Drive | 5500W Shihlin Frequency Inverter |
| Circular Cutter Motor | Three-phase asynchronous motor/ MODOLI brand |
| Circular Cutter Drive | 3700W Shihlin Frequency Inverter |
| Cross Travel Servo Motor | Shihlin Servo Motor |
| Cross Travel Servo Driver | Shihlin Servo Driver |
| Cutter Feed Servo Motor | Shihlin Servo Motor |
| Cutter Feed Servo Driver | Shihlin Servo Driver |
| Swing Angle Stepper Motor | Domestic brand |
| High-Speed Steel Cutter | SANSHI brand |

| Accessory Name | Brand / Specification |
| --- | --- |
| Air Circuit Breaker | CHINT |
| Switching Power Supply | TEND (Taiwan) |
| Photoelectric Sensor | Domestic brand |
| Proximity Switch | Domestic brand |
| AC Contactor | CHINT |
| Relay | OMRON |
| Push Button Switch | Domestic brand |
| Cutter Feed Ball Screw | HIWIN |
| Cross Travel Ball Screw | HIWIN |
| Swing Angle Ball Screw | HIWIN |
| Cutter Feed Linear Guide | HIWIN |
| Cross Travel Linear Guide | HIWIN |

Configuration parameters：

| Accessory Name | Brand / Specification Description |
| --- | --- |
| Solenoid Valve | Domestic brand |
| Air Cylinder | Domestic brand |
| Linear Guide Rail | HIWIN |
| Magnetic Switch | Domestic brand |
| Main Spindle (4 pcs) | Custom-made |
| Bearing | NSK |
| Machine Frame | Imported baking paint finish |
| Circular Cutters (Single-sided / Double-sided, high-speed steel / tungsten steel) | 4 pcs (selected according to processing materials) |
| Tool Kit | 1 set |
| Dial Indicator | 1 set |
| Cutter Alignment Rod | 1 pc |
| PE Tubing | 4 pcs |