- Rewinding Coding Machine ZRD-220

Basic Use：

- Mainly used for laser coding and rewinding of various coiled materials.

- Equipment Advantages：

- Human-machine interface with microcomputer programmable control.

- Equipped with three-servo control system. Servo motors control rewinding and unwinding tension to ensure stable and reliable material tension; servo positioning film feeding with adjustable length.

- Adopts high-precision water-cooled ultraviolet laser.

- Adjustable coding quantity for products.

- Easy operation, automatic fault alarm and one-key intelligent operation.

- Main Technical Parameters：

| - Model | - ZRD-220 Rewinding Coding Machine |
| --- | --- |
| - Material Width | - 50-220mm |
| - Material Diameter | - Max outer diameter: 245mm (core diameter: 76mm) |
| - Coding Speed | - Depends on coding requirements |
| - Mechanical Allowable Speed | - 0-30m/min |
| - Laser Type | - 10W UV Laser (355nm) |
| - Voltage and Power | - AC220V 2.6KW |
| - Air Pressure | - 0.5mpa |
| - Machine Dimensions | - 1620mm*1380mm*1750mm |
| - Net Weight | - 1600KG |

- Configuration Parameters:

| - Model | Rewinding Coding Machine ZRD-220 |
| --- | --- |
| - Touch Screen | - HMI |
| - Servo Roll Feed | - HCFA |
| - Roll Feeding Driver | - HCFA |
| - Fixed-length Servo | - HCFA |
| - Fixed-length Driver | - HCFA |
| - Rewinding Servo | - HCFA |
| - Rewinding Driver | - HCFA |
| - Laser Coding Machine | - KEXIN |
| - Laser Galvo Scanner | - KEXIN |
| - Reducer | - Smart |
| - Detection Fiber | - KEYENCE |
| - PLC | - HCFA |
| - Air Switch | - CHINT |
| - Switching Power Supply | - MEAN WELL |
| - Deviation Correction | - Youdao |
| - Relay | - IDEC |
| - Push Button Switch | - Schneider |
| - Cylinder | - XINGCHEN |
| - Filter | - STNC |
| - Solenoid Valve | - STNC |
| - Magnetic Switch | - AirTAC |
| - Bearing | - NSK |
| - Frame | - Imported Powder Coating |
| - Motion Controller | - HCFA |
| - Photoelectric Sensor | - Boyee |
| - Guide Rail | - HIWIN |