Dual Shaft Rewinding Machine ZRFJ-1300-D250

Equipment Layout：

Basic Use：

Mainly used for adhesive rewinding of automotive foam, foam double-sided tape, BOPP tape, masking tape, double-sided tape and other materials.

Equipment Advantages:

- Equipped with Hechuan motion control system to realize automatic taper tension. Different tension is generated according to material thickness and winding diameter during rewinding.

- The active traction adopts 5.5kW servo control. The rewinding length accuracy is within 5mm-10mm. It is applicable to unwinding with high/low tension and enables continuous, long-term stable operation.

- Automatically shifting electronic pressure roller: automatically adjusts following the set material thickness, material thickness variation and length change. It maintains the gap between pressure roller and winding shaft, delivering flatter finished rolls with consistent tension tightness for various roll diameters.

- Automatic tension winding & unwinding with stable tension and smooth start-stop. Servo motor torque winding ensures stable and precise tension for neat rolls with flat end faces.

- The glue roller surface adopts nano-coating; all back-up rollers are mirror finished to avoid scratching materials. All pressure rollers are made of silicone rubber.

- Curved spread roller flattens various materials, enabling efficient high-speed rewinding.

- Dual-shaft winding with roll flipping function; manual cutting and tape splicing. The tail pressing roller automatically flattens materials to improve productivity effectively.

- The material suction worktable features automatic suction during manual cutting, facilitating material joining and tape splicing.

- 10-inch touch screen for simple and intuitive operation.

- Working width can be customized according to materials, models available: 1100, 1300, 1600, etc.

- Main Technical Parameters：

| - Model | - ZRFJ-1300-D250 |
| --- | --- |
| - Paper core | - Customized as customer requirements (Interchangeable) - 25/38/50/76 |
| - Unwinding master roll | - Max. Diameter 800mm |
| - Rewinding diameter | - 250mm |
| - Rewind width | - 1300mm |
| - Production speed | - Line Speed200m/min - 4-5pcs/min（15m） - Subject to rewinding length |
| - Power supply | - 380V 50/60HZ/18KW |
| - Machine size | - 2800*2500*1600mm |
| - Net Weight | - 1.2T |

- Configuration Parameters：

| - Model | - ZRF-1300-D200 |  |
| --- | --- | --- |
| - Touch Screen | - 10-inch MCGS |  |
| - Frame Wall Plate | - 25mm |  |
| - Motion Controller | - HCFA |  |
| - Active Traction Servo Motor | - 5.5kwHCFA |  |
| - Active Traction Servo Driver | - 5.5kwHCFA |  |
| - Winding Pressure Roller Servo Motor | - 1.5kwHCFA |  |
| - Winding Pressure Roller Driver | - 1.5kwHCFA |  |
| - Winding Servo Motor | - 4.4kwHCFA |  |
| - Winding Servo Driver | - 4.4kwHCFA |  |
| - Roll Flipping Servo Motor | - 1kwHCFA |  |
| - Roll Flipping Servo Driver | - 1kwHCFA |  |
| - Tail Winding Servo Motor | - 1kwHCFA |  |
| - Tail Winding Servo Driver | - 1kwHCFA |  |
| - Key-type Unwinding Air Shaft | - 3inch |  |
| - Plate-type Winding Air Shaft | - 3inches |  |
| - Motor | - MaiLi |  |
| - Planetary Reducer | - Smetre |  |
| - Magnetic Powder Brake | - WREZUN |  |
| - Air Circuit Breaker | - chint |  |
| - Switching Power Supply | - Mingwei |  |
| - Photoelectric Sensor | - BOJKE |  |
| - Proximity Switch | - Yangming |  |
| - AC Contactor | - Schneider |  |
| - Relay | - IDEA |  |
| - Push Button Switch | - Schneider |  |
| - 3-color Warning Light | - TAYB |  |
| - Solenoid Valve | - ASICKO |  |
| - Air Cylinder | - AIRTAC |  |
| - Linear Guide Rail | - HIWIN |  |
| - Magnetic Switch | - AIRTAC |  |
| - 3-inch Key-type Unwinding Air Shaft | - Taiwan |  |
| - Bearing | - NSK |  |
| - Dual Traction Main Shaft | - Nano-coating Anti-sticking Treatment |  |
| - Winding Pressure Roller | - Φ180mm Silicone Pressure Roller |  |
| - Transition Roller | - Hard Anodizing, Brown Finish |  |
| - Tail Pressure Roller | - Φ80mm Silicone Pressure Roller |  |
| - Traction Pressure Roller | - Φ80mm Silicone Pressure Roller |  |
| - Machine Body | - Spray Coating & Baking Finish |  |